- MTU & MSS .  in which payload will paas.

1. MTU (Maximum Transmission Unit)

Definition: Maximum size of a packet that can be sent over a network including headers + payload

Example:

Ethernet MTU = 1500 bytes

It includes:

IP header (20 bytes)

TCP header (20 bytes)

Payload (actual data)

So MTU = Header + Data (Payload)

2. MSS (Maximum Segment Size)

Definition: Maximum size of only the TCP data (payload) that can be sent in a single segment

It does NOT include headers

Formula:

MSS = MTU - IP Header - TCP Header

Example

MTU = 1500

IP Header = 20 bytes

TCP Header = 20 bytes

MSS = 1500 - 20 - 20 = 1460 bytes

Solution:

tmsh modify ltm profile tcp my_tcp_profile mss 1360

tmsh modify ltm virtual my_vs profiles add { my_tcp_profile }

--------------------------------------------------------------------------------------------------------------------------------------

- 3-way handshake. What information will pass in syn packet.

Steps:

SYN (Client → Server)

SYN-ACK (Server → Client)

ACK (Client → Server)

SYN packet carries:

Sequence Number

MSS

Window Size

Window Scaling

SACK Permitted

Timestamps

Ports + IP Info

- 4 way handshake .

Steps in 4-Way Handshake

1. FIN (Client → Server)

Client sends FIN = 1

Means: “I have finished sending data”

2. ACK (Server → Client)

Server sends ACK

Acknowledges client’s FIN
 Connection becomes half-closed

3. FIN (Server → Client)

Server sends its own FIN

Means: “Now I’m also done sending data”

4. ACK (Client → Server)

Client sends final ACK

Connection fully closed

- What is tcp windowing . What will happen when window is small , will packet drop ?

“TCP window size tells the sender how much data the receiver can accept at a time.”

NO (normally packets do NOT drop due to small window)

- Small window = flow control

- Packet drop = congestion or errors

- What is selective ack .

Selective Acknowledgment (SACK) is a TCP feature that allows the receiver to inform the sender exactly which data segments were received successfully.

Instead of acknowledging only the last in-order byte, it tells:
“I received these specific chunks, but these are missing.”

With SACK

- Receiver sends:

  - “I got packet 1, 2, 4, 5… missing 3”

- Sender retransmits only packet 3

By default it is enabled in F5 but if want enabled in old F5 Device bwlow is command

create ltm profile tcp Abcdbc time-wait-timeout  1500 selective-acks enabled

- LSA 5

LSA Type 5 = External LSA in OSPF

It is used to advertise routes coming from outside the OSPF domain

“LSA Type 5 is used by ASBR to advertise external routes into OSPF.”

ASBR (Autonomous System Boundary Router)

If area is:

Stub / Totally Stubby
❌ LSA 5 blocked

  Alternative used:
👉 Default route instead

- How to check if application is not opening to client --- what troubleshooting steps

Step-by-Step Troubleshooting

1. DNS Check (First step)

nslookup app.example.com

✔ Verify:

Correct VIP/IP is resolving

No wrong DNS entry

2. Basic Connectivity (Client → VIP)

ping <VIP>
telnet <VIP> 80
telnet <VIP> 443

👉 Results:

❌ Ping fail → network issue

❌ Port fail → firewall / service issue

3. Check ADC (Load Balancer)

On F5 Networks

✔ Virtual Server

tmsh show ltm virtual <vs_name>

Must be Available (green)

✔ Pool Members

tmsh show ltm pool <pool_name>

All members should be UP

If pool down → app won’t open

On Citrix

show lb vserver <vserver>
show service <service>

4. Check if Traffic Reaches ADC 🔥

tcpdump -i 0.0:nnn host <client_ip>

Analyze:

No SYN → issue before ADC

SYN comes but no response → ADC/server issue

5. TCP Handshake Analysis (Very Important)

| Scenario | Meaning |
| --- | --- |
| No SYN | Client/network issue |
| SYN → No SYN-ACK | Server or pool issue |
| SYN → SYN-ACK → No ACK | Firewall / client issue |
| Handshake OK but no response | Application issue |

6. Check Server Connectivity (ADC → Server)

ping <server_ip>
telnet <server_ip> <port>

If fails → backend unreachable

7. Check Application on Server

curl http://localhost

✔ Verify:

App is running

Port is listening

8. SSL Troubleshooting (if HTTPS)

openssl s_client -connect <VIP>:443

Check:

Certificate

Handshake errors

9. Check Logs

F5:

tail -f /var/log/ltm

Look for:

Reset (RST)

SSL errors

iRule issues

10. Firewall / Security Check

Is port allowed?

Any blocking rule?

WAF/ASM blocking?

11. Performance / Resource Check

Server CPU / Memory

Application threads

DB connectivity

- What is ssl handshake . packet flow .

“SSL handshake is a process where client and server establish a secure encrypted connection by exchanging certificates, keys, and algorithms.”

“SSL handshake is the process where client and server exchange certificates, agree on cipher suites, and generate session keys to establish a secure encrypted connection before data transfer”

1. Client Hello (Client → Server)

- Supported TLS versions

- Cipher suites

- Random number (Client Random)

- Extensions (SNI, ALPN)

🔸 2. Server Hello (Server → Client)

- Selected TLS version

- Selected cipher suite

- Server Random

🔸 3. Server Certificate

- Server sends its digital certificate

- Contains:

  - Public key

  - Issuer (CA)

🔸 4. Server Hello Done

- Server says: “I’m done with initial negotiation”

🔸 5. Client Key Exchange

- Client generates Pre-Master Secret

- Encrypts it using server’s public key

- Sends to server

🔸 6. Session Key Generation

- Both sides generate:

  - Same Session Keys

- Using:

  - Client Random + Server Random + Pre-Master Secret

🔸 7. Change Cipher Spec

- Client & server switch to encrypted communication

🔸 8. Finished Message

- Both send encrypted “Finished” message
👉 Handshake complete ✔️

- When client will send certificate, what F5 will do…mutual auth question.

Step-by-step: What F5 does when client sends certificate

Client sends Certificate

During SSL handshake, after Server Hello, F5 requests client certificate (if enabled).

Client sends its certificate to F5.

F5 validates the Client Certificate

F5 checks:

✅ Certificate Chain Validation

Verifies if the client certificate is signed by a trusted CA (Certificate Authority) configured in F5.

Uses the Trusted Certificate Authorities bundle in SSL profile.

✅ Certificate Expiry

Checks if certificate is still valid (not expired).

✅ Revocation Check (Optional)

CRL (Certificate Revocation List)

OCSP (Online Certificate Status Protocol)

✅ Certificate Signature

Ensures certificate is not tampered.

Client Certificate Request Setting (Important in F5)

In Client SSL Profile:

Ignore → F5 accepts connection even if client doesn't send cert

Request → F5 asks but does not enforce

Require → ❗ Strict mTLS (handshake fails if no valid cert)

If validation is successful

SSL handshake continues

Connection is established

F5 can pass client cert details to backend (via headers like X-SSL-Client-Cert using iRules)

If validation fails

F5 immediately terminates the handshake

Client gets SSL error (handshake failure)

- Complete process to DNS to backend IP.

- Iterative and recursive lookup

- Vpc peering in aws

- Paramiko and Netmiko difference

- Packet flow of firewall . when source nat and destination nat will happen

- On which hardware did you work and current software

- What hardware parameter you check when migration from one box to another

- Ucs and scf difference . how to upload back in diifernt box and wht are things need to check before upload

- F5 upgrade process

- Why we need relicensing in upgrade

- Can we add add-on license

Yes — in F5 BIG-IP, you can add an add-on license, but it depends on what exactly you mean by “add-on.” Let me clarify clearly.

1. What is an Add-On License in F5?

An add-on license is used to enable additional modules or features on an already licensed BIG-IP device.

Examples:

Adding ASM (WAF) to an existing LTM device

Adding APM, AFM, DNS (GTM), etc.

Increasing throughput (e.g., VE upgrade from 200 Mbps → 1 Gbps)

2. Can We Add It Later?

Yes, absolutely.
F5 supports license expansion, so you don’t need to rebuild the device.

3. How to Add an Add-On License.

  Go to:

System → License → Activate

Choose:

- Add-on / Modify License

Enter:

- New Base Registration Key (if provided) OR

- Additional add-on key

Click Next → Accept → Activate

Method 2: CLI

tmsh install sys license registration-key <new-key>

tmsh load sys config

- What detail we use in monitor

| Parameter | Description | Example |
| --- | --- | --- |
| Type | Defines protocol used for health check | HTTP, HTTPS, TCP, ICMP |
| Interval | Time between each health check | 5 seconds |
| Timeout | Time to wait before marking server DOWN | 16 seconds (3 × interval + 1) |
| Send String | Request sent to server | GET /healthcheck.html HTTP/1.1\r\nHost: example.com\r\n\r\n |
| Receive String | Expected response from server | 200 OK |
| Reverse | Marks server DOWN if response is received | Enabled/Disabled |
| Destination | Target IP/Port for monitor | Node IP:Port or custom IP |
| Time Until Up | Delay before marking server UP after recovery | 10 seconds |
| Manual Resume | Requires manual action to bring node back UP | Enabled/Disabled |
| Transparent | Monitors a different IP via pool member | Enabled (used in routed scenarios) |

- What will u check if pool is down with a monitor

| Check Point | What to Verify | Why It Matters |
| --- | --- | --- |
| Monitor Status | Check if monitor shows DOWN (red) | Confirms issue is due to health check, not config |
| Monitor Type | Verify correct type (HTTP vs TCP vs HTTPS) | Wrong type → false DOWN status |
| Send String | Validate request format (URL, headers, syntax) | Incorrect request → server won’t respond properly |
| Receive String | Check expected response string | Mismatch → server marked DOWN even if working |
| Application Response | Test manually using curl/telnet from F5 | Confirms whether app is actually reachable |
| Timeout & Interval | Verify values (Timeout = 3×Interval+1) | Too low → false failures |
| Port/Service | Check if correct port is configured | Wrong port → no response |
| Server Health | Check if application/service is running on backend | App down = monitor fails |
| Network Connectivity | Ping / telnet from F5 to server | Routing/firewall issues can block traffic |
| SSL Settings (HTTPS) | Check certificates / SSL handshake | SSL errors → monitor fails |
| Firewall / ACL | Ensure F5 IP is allowed on backend | Blocked traffic → no response |
| Reverse Option | Verify if enabled incorrectly | Can mark server DOWN when it’s actually UP |
| Manual Resume | Check if enabled | Node won’t come UP automatically |
| Logs | Check /var/log/ltm | Shows exact reason for failure |
| Monitor Assignment | Ensure correct monitor is attached to pool/member | Wrong monitor → incorrect health check |

- https monitor marking node down .what could be the reason and how to check

| Reason | What Happens | How to Check / Fix |
| --- | --- | --- |
| SSL Handshake Failure | Monitor cannot establish SSL connection | Use openssl s_client -connect <IP:443> from F5 bash |
| Certificate Issue | Expired / invalid / untrusted cert | Check server certificate validity |
| Wrong Monitor Type | Using HTTP monitor on HTTPS port | Ensure monitor type = HTTPS |
| Send String Incorrect | Bad request format (missing headers, wrong URL) | Validate HTTP GET format with proper Host header |
| Receive String Mismatch | Expected string not found in response | Check actual response using curl |
| TLS Version Mismatch | Server supports only TLS 1.2/1.3 but monitor uses different | Configure monitor SSL profile or test via OpenSSL |
| Cipher Mismatch | No common cipher between F5 and server | Check using openssl and adjust SSL profile |
| SNI Issue | Server requires SNI but monitor doesn’t send it | Configure Server Name in monitor |
| Firewall Blocking | F5 traffic blocked by backend firewall | Test using telnet/curl from F5 |
| Port Issue | Wrong port configured | Verify correct port (usually 443) |
| Application Issue | App returns error page (500, 404, etc.) | Check via curl/browser |
| Reverse Option Enabled | Marks DOWN when response is received | Disable reverse if not required |

- how to configure GTM

Key Components in GTM

| Component | Purpose |
| --- | --- |
| Data Center | Logical grouping of servers/sites |
| Server | Represents BIG-IP or backend server |
| Virtual Server | Actual application VIP (LTM VIP) |
| Pool | Group of virtual servers |
| Wide IP | Public DNS name mapped to pool |
| Monitor | Health check for servers/VIPs |

Step-by-Step Configuration

1. Create Data Center

Path:

DNS → GSLB → Data Centers → Create

Name: DC_Pune

Location: Optional

2. Add Server

Path:

DNS → GSLB → Servers → Create

Name: Server_Pune

IP Address: LTM Self-IP / Management IP

Data Center: DC_Pune

Choose:

BIG-IP System (if integrating with LTM)

3. Add Virtual Server

Automatically discovered (if iQuery working)

OR manually:

DNS → GSLB → Servers → Virtual Servers → Add

IP: VIP IP (from LTM)

Port: 80 / 443

4. Create GTM Pool

Path:

DNS → GSLB → Pools → Create

Name: web_pool

Type: A (for IPv4)

Add Members:

Virtual Server 1

Virtual Server 2 (DR site)

Load Balancing Method:

Round Robin / Ratio / Topology

5. Create Wide IP (Main Step)

Path:

DNS → GSLB → Wide IPs → Create

Name: www.example.com

Pool: web_pool

This is what client resolves via DNS

6. Configure Health Monitor

Attach monitor to:

Pool OR

Virtual Server

Example:

HTTP / HTTPS monitor

7. Configure iQuery (Important)

Required for GTM ↔ LTM communication

Check:

tmsh show gtm iquery

Port used: 4353

8. DNS Configuration

Point domain to GTM (BIG-IP DNS)

Update NS records or delegate zone

- what is wide ip

A Wide IP in F5 is a DNS name (FQDN) that maps to one or more GTM pools,

which in turn contain virtual servers (VIPs).

Wide IP = DNS entry + intelligent traffic distribution

Structure of Wide IP

| Component | Description |
| --- | --- |
| Wide IP | Public DNS name (e.g., www.example.com) |
| Pool | Group of virtual servers |
| Virtual Server | LTM VIPs (actual application endpoints) |
| Monitor | Health check to decide availability |

- How to configure wide ip

Step-by-Step Configuration

Step 1: Create GTM Pool

DNS → GSLB → Pools → Create

Name: web_pool

Type: A (IPv4)

Add Members:

Select Virtual Servers (VIPs)

Load Balancing Method:

Round Robin / Ratio / Global Availability

Step 2: Create Wide IP (Main Step)

DNS → GSLB → Wide IPs → Create

Name (FQDN): www.example.com

Type: A (IPv4)

Pools: Add web_pool

Step 3: Configure Load Balancing

At Wide IP level:

LB Method:

  - Round Robin / Ratio / Topology

Step 4: Attach Health Monitor

Assign monitor to:

  - Pool OR

  - Virtual Server

Example:

HTTP / HTTPS monitor

Step 5: DNS Delegation

Point your domain to GTM:

  - Update NS records to GTM (BIG-IP DNS

Step 6: Verify Configuration

tmsh list gtm wideip

nslookup www.example.com

- what will check if traffic is not failover to secondary member when we disable primary member in wide ip config

| Check Point | What to Verify | Why It Matters |
| --- | --- | --- |
| Primary Member Status | Ensure primary VIP/member is actually marked DOWN (not just disabled incorrectly) | GTM shifts traffic only when it detects member DOWN |
| Monitor Status | Check if health monitor is marking primary DOWN | Without monitor failure → GTM still thinks primary is UP |
| Manual Disable vs Force Down | Difference between “Disabled” and “Forced Offline” | Disabled = existing connections allowed; Forced Offline = immediate failover |
| Load Balancing Method | Verify LB method (Global Availability, Ratio, Round Robin) | Failover behavior depends on LB method |
| Pool Order (Priority) | Check priority/group order in pool | Wrong order → traffic may not go to secondary |
| Fallback IP | Check if fallback IP is configured | GTM may return fallback instead of secondary |
| Persistence (LDNS) | Check if DNS persistence is enabled | Client may still resolve to old (primary) IP |
| DNS TTL | Verify TTL value of Wide IP | High TTL → client keeps using cached IP |
| DNS Cache (Client/Local) | Clear DNS cache on client / local DNS server | Cached response prevents failover |
| iQuery Communication | Ensure GTM ↔ LTM communication is working | Without iQuery, GTM won’t know member status |
| Virtual Server Status | Check if VIP is really DOWN at LTM level | If VIP is UP → GTM won’t failover |
| Logs | Check GTM logs (/var/log/gtm, /var/log/ltm) | Helps identify decision-making issues |

- GTM config check

| Component | What to Check | Command / Path |
| --- | --- | --- |
| License | GTM (DNS) module is licensed | tmsh show sys license |
| Provisioning | DNS module is enabled | tmsh show sys provision |
| Data Center | Data centers created and active | tmsh list gtm datacenter |
| Server | Servers added and associated with data center | tmsh list gtm server |
| iQuery Status | GTM ↔ LTM communication working | tmsh show gtm iquery |
| Virtual Servers | VIPs discovered and showing UP | tmsh list gtm server <server-name> virtual-servers |
| GTM Pool | Pools created with correct members | tmsh list gtm pool |
| Pool Member Status | Members showing UP/DOWN correctly | tmsh show gtm pool |
| Wide IP | Wide IP configured and linked to pool | tmsh list gtm wideip |
| Load Balancing Method | Correct LB method (Round Robin / GA / Ratio) | Check in pool / Wide IP config |
| Monitor | Health monitors attached and working | tmsh list ltm monitor / GUI |
| DNS Listener | GTM listening on port 53 | tmsh list ltm dns / check listener |
| DNS Resolution | Domain resolving correctly | nslookup www.example.com |
| TTL Value | TTL configured properly | Check Wide IP settings |
| Persistence | LDNS persistence settings | Check Wide IP config |
| Logs | Errors in GTM/LTM logs | /var/log/gtm, /var/log/ltm |

  - Check iQuery tmsh show gtm iquery

  - Check Pool & Member Status  tmsh show gtm pool

  - Check Wide IP  tmsh show gtm wideip

  - Test DNS Resolution nslookup www.example.com

  - 5. Check Logs  tail -f /var/log/gtm

- LTM config check

LTM Configuration Check (F5 BIG-IP LTM)

| Component | What to Check | Command / Path |
| --- | --- | --- |
| License | LTM module is licensed | tmsh show sys license |
| Provisioning | LTM is provisioned | tmsh show sys provision |
| VLANs | Correct VLANs configured | tmsh list net vlan |
| Self IPs | Self IPs assigned and reachable | tmsh list net self |
| Routing | Default route / static routes | tmsh list net route |
| SNAT | SNAT Automap / SNAT Pool configured | Check Virtual Server / tmsh list ltm snat |
| Nodes | Backend server IPs added | tmsh list ltm node |
| Pool | Pool created with correct members | tmsh list ltm pool |
| Pool Member Status | Members showing UP/DOWN | tmsh show ltm pool |
| Monitor | Health monitors attached and working | tmsh list ltm monitor |
| Virtual Server (VIP) | VIP configured with correct IP/port | tmsh list ltm virtual |
| Profiles | HTTP, TCP, SSL profiles attached | Check in Virtual Server config |
| SSL Config | Certificates, keys, SSL profiles | tmsh list ltm profile client-ssl |
| Persistence | Session persistence configured | Cookie / Source IP |
| iRules (if any) | Custom logic applied correctly | tmsh list ltm rule |
| NAT / Firewall | Ensure no blocking between client ↔ F5 ↔ server | Network/firewall check |
| Logs | Errors in logs | /var/log/ltm |

- Is Ansible pull or push technology

Push vs Pull in Ansible

| Type | Explanation |
| --- | --- |
| Push (Default) | Control node (Ansible server) connects to target nodes and pushes configurations using SSH |
| Pull (Optional) | Target nodes pull configuration from a central repository using ansible-pull |

- What protocol and port Ansible use and in which language it is written

Ansible – Protocol, Port & Language

| Parameter | Answer |
| --- | --- |
| Protocol | SSH (Linux/Unix), WinRM (Windows) |
| Port (Linux) | 22 (SSH default) |
| Port (Windows) | 5985 (HTTP), 5986 (HTTPS for WinRM) |
| Language | Python |

- What is ssl handshake

An SSL/TLS handshake is the process where a client and server:

- Authenticate each other

- Agree on encryption algorithms

- Exchange keys

- Establish a secure encrypted connection

| Step | Message | Description |
| --- | --- | --- |
| 1 | Client Hello | Client sends supported TLS versions, cipher suites, random number |
| 2 | Server Hello | Server selects TLS version, cipher suite, sends its random number |
| 3 | Certificate | Server sends its SSL certificate (for identity verification) |
| 4 | Server Key Exchange | Sends key info (if required) |
| 5 | Server Hello Done | Server indicates it finished its part |
| 6 | Client Key Exchange | Client sends pre-master secret (encrypted with server public key) |
| 7 | Change Cipher Spec | Client switches to encrypted communication |
| 8 | Finished (Client) | Client sends encrypted message |
| 9 | Change Cipher Spec | Server switches to encrypted communication |
| 10 | Finished (Server) | Server sends encrypted message → handshake complete |

- What are parameters in client hello in ssl handshake

Client Hello is the first message sent by the client to initiate an SSL/TLS handshake. It contains multiple parameters that help the server decide how to establish a secure session.

| Parameter | Description |
| --- | --- |
| TLS Version | Highest TLS version supported by client (e.g., TLS 1.2, TLS 1.3) |
| Client Random | Random number used in key generation |
| Session ID | Used for session resumption (reuse previous session) |
| Cipher Suites | List of supported encryption algorithms |
| Compression Methods | Supported compression (usually none nowadays) |
| Extensions | Additional capabilities and options |

- What is pre-master key in ssl handshake

The Pre-Master Key (Pre-Master Secret) is a temporary secret generated during the SSL/TLS handshake, which is later used to derive the Master Key (Master Secret) for encrypting the session

| Aspect | Explanation |
| --- | --- |
| Who generates it | Client (in most cases like RSA handshake) |
| Purpose | Used to generate the Master Secret |
| Encryption | Encrypted using server’s public key |
| Security | Ensures only server (with private key) can decrypt it |
| Lifetime | Temporary (used only during handshake) |

- Is is symmetric or asymmetric in ssl handshake

Asymmetric Encryption (During Handshake)

Uses:

Public key (server certificate)

Private key (server side)

Used for:

Securely exchanging Pre-Master Key

Authentication

Example:

Client encrypts pre-master key using server’s public key

Symmetric Encryption (After Handshake)

Both client & server generate same session key

Used for:

Encrypting actual application data (HTTP, API, etc.)

Faster than asymmetric → used for bulk data transfer

- Ipsec phase 1 & phase 2 parameter and messages

📊 Phase 1 (IKE Phase 1)

✅ Purpose

- Establish secure control channel (ISAKMP SA)

- Authenticate peers

- Create secure tunnel for Phase 2

📊 Phase 1 Parameters

| Parameter | Description |
| --- | --- |
| Encryption Algorithm | AES, 3DES |
| Hash Algorithm | SHA-1, SHA-256 |
| Authentication Method | Pre-Shared Key (PSK) or Certificates |
| DH Group | Diffie-Hellman group (Group 2, 5, 14, etc.) |
| Lifetime | Duration of Phase 1 SA |
| Mode | Main Mode / Aggressive Mode |
| Peer IP | Remote VPN peer address |

📩 Phase 1 Messages

🔹 Main Mode (6 Messages)

| Step | Message |
| --- | --- |
| 1 | SA Proposal |
| 2 | SA Selection |
| 3 | Key Exchange (DH) |
| 4 | Key Exchange Reply |
| 5 | Authentication (ID, Hash) |
| 6 | Authentication Confirm |

🔹 Aggressive Mode (3 Messages)

| Step | Message |
| --- | --- |
| 1 | SA + Key Exchange + ID |
| 2 | SA + Key Exchange + ID + Auth |
| 3 | Authentication Confirm |

📊 Phase 2 (IKE Phase 2)

Establish IPsec SA

Define how actual data will be encrypted

📊 Phase 2 Parameters

| Parameter | Description |
| --- | --- |
| Protocol | ESP (most common) / AH |
| Encryption Algorithm | AES, 3DES |
| Hash Algorithm | SHA-1, SHA-256 |
| PFS (Perfect Forward Secrecy) | Optional DH group for extra security |
| Lifetime | Duration of Phase 2 SA |
| Local/Remote Subnets | Interesting traffic (e.g., 10.1.1.0/24 ↔ 10.2.2.0/24) |

📩 Phase 2 Messages (Quick Mode – 3 Messages)

| Step | Message |
| --- | --- |
| 1 | SA Proposal + Nonce |
| 2 | SA Selection + Nonce |
| 3 | Confirmation |

🔄 Summary Difference

| Aspect | Phase 1 | Phase 2 |
| --- | --- | --- |
| Purpose | Secure channel (ISAKMP SA) | Data encryption (IPsec SA) |
| Mode | Main / Aggressive | Quick Mode |
| Messages | 6 (Main), 3 (Aggressive) | 3 |
| Encryption Scope | Control plane | Data plane |

- What is root and intermediate ca in cert authority

| Aspect | Root CA | Intermediate CA |
| --- | --- | --- |
| Definition | Top-level trusted authority | Subordinate CA issued by Root CA |
| Certificate Type | Self-signed certificate | Signed by Root CA (or another intermediate) |
| Trust Level | Directly trusted by browsers/OS | Trusted through Root CA |
| Security | Kept highly secure (offline usually) | Used for day-to-day certificate signing |
| Purpose | Establish trust anchor | Issue certificates to servers/clients |
| Exposure | Rarely exposed | Frequently used |

Certificate Chain - Root CA → Intermediate CA → Server Certificate

- Source to destination to able to connect what will check in firewall if policy is there and traffic is passing and still destination not reachable

| Check Point | What to Verify | Why It Matters |
| --- | --- | --- |
| Security Policy | Rule allows correct Source, Destination, Port, Protocol | Wrong object/group → traffic mismatch |
| Hit Count | Policy hit count increasing | Confirms traffic is actually matching rule |
| NAT Configuration | Source NAT / Destination NAT correct | Wrong NAT → destination unreachable or wrong IP |
| Routing (Firewall) | Correct route to destination network | No route → packet drop |
| Return Path | Destination can route back to source | Asymmetric routing → response never comes back |
| Firewall Logs | Allow/Deny logs | May show silent drops or different rule match |
| Service/Port | Correct port open (e.g., 80, 443, 22) | Wrong port → connection fails |
| Application Running | Service actually running on destination | Firewall OK but app down |
| Zone/Interface | Correct source & destination zones | Wrong zone → policy not applied |
| Stateful Inspection | Session created or not | No session → traffic not established |
| IPS/AV/UTM | Security profiles blocking traffic | Threat detection may block silently |
| Packet Drops | Check drop counters | Hardware/software drops |
| MTU / Fragmentation | Packet size issues | Large packets may drop |
| ARP / MAC Resolution | Firewall can resolve destination MAC | ARP failure → no forwarding |

- What is irule

An iRule is a custom script written in TCL (Tool Command Language) used in F5 BIG-IP LTM to control and manipulate traffic.

It allows you to apply advanced logic beyond standard configuration.

| Aspect | Details |
| --- | --- |
| Language | TCL |
| Used In | F5 BIG-IP (LTM, ASM, etc.) |
| Purpose | Traffic manipulation & decision-making |
| Execution Point | Applied on Virtual Server |
| Events Based | Executes on events (e.g., request, response, connection) |

- What are the events ?

In F5 iRules, events are predefined triggers that tell the BIG-IP system when to execute a block of iRule code. Each event corresponds to a specific stage in the connection lifecycle (client-side, server-side, HTTP processing, SSL handshake, etc.).

| Category | Event Name | When it Triggers |
| --- | --- | --- |
| Connection (L4) | CLIENT_ACCEPTED | When client connects to F5 |
|  | CLIENT_CLOSED | When client disconnects |
|  | SERVER_CONNECTED | When F5 connects to server |
|  | SERVER_CLOSED | When server disconnects |
| ------------------------ | ----------------------------- | ---------------------- |
| HTTP (L7) | HTTP_REQUEST | When HTTP request is received |
|  | HTTP_REQUEST_DATA | When request body is collected |
|  | HTTP_RESPONSE | When response is received from server |
|  | HTTP_RESPONSE_DATA | When response body is collected |
|  | HTTP_REQUEST_RELEASE | Before sending request to server |
|  | HTTP_RESPONSE_RELEASE | Before sending response to client |
| ------------------------ | ----------------------------- | ---------------------- |
| SSL | CLIENTSSL_HANDSHAKE | During client SSL handshake |
|  | CLIENTSSL_CLIENTCERT | When client sends certificate |
|  | SERVERSSL_HANDSHAKE | During server SSL handshake |
| ------------------------ | ----------------------------- | ---------------------- |
| Load Balancing | LB_SELECTED | When pool member is selected |
|  | LB_FAILED | When no pool member available |
| ------------------------ | ----------------------------- | ---------------------- |
| DNS | DNS_REQUEST | When DNS query is received |
|  | DNS_RESPONSE | When DNS response is sent |
| ------------------------ | ----------------------------- | ---------------------- |
| Persistence | PERSIST_DOWN | When persistence record fails |
| ------------------------ | ----------------------------- | ---------------------- |
| APM (Access) | ACCESS_SESSION_STARTED | When session starts |
|  | ACCESS_POLICY_COMPLETED | When policy completes |
| ------------------------ | ----------------------------- | ---------------------- |
| Other | RULE_INIT | When iRule is loaded |
|  | NAME_RESOLVED | When DNS lookup completes |

- Example of events

1. CLIENT_ACCEPTED (Connection Start)

Log client IP when connection starts

when CLIENT_ACCEPTED {

log local0. "Client connected: [IP::client_addr]"

}

2. HTTP_REQUEST (Request Handling)

Redirect HTTP to HTTPS

when HTTP_REQUEST {

if {[HTTP::host] eq "example.com"} {

HTTP::redirect "https://[HTTP::host][HTTP::uri]"

}

}

3. HTTP_REQUEST (URL-based Routing)

Send traffic to different pool

when HTTP_REQUEST {

if {[HTTP::uri] starts_with "/api"} {

pool api_pool

} else {

pool web_pool

}

}

4. HTTP_RESPONSE (Modify Response)

Add security header

when HTTP_RESPONSE {

HTTP::header insert "X-Security" "Enabled"

}

5. CLIENTSSL_CLIENTCERT (Mutual Authentication)

Check if client certificate exists

when CLIENTSSL_CLIENTCERT {

if {[SSL::cert count] < 1} {

reject

}

}

6. LB_FAILED (Failover Scenario)

Handle when no server is available

when LB_FAILED {

HTTP::respond 503 content "Service Unavailable"

}

7. SERVER_CONNECTED (Server Side Logging)

Log selected backend server

when SERVER_CONNECTED {

log local0. "Connected to server: [IP::server_addr]"

}

8. RULE_INIT (Initialization)

Define static variable

when RULE_INIT {

set static::app_name "MyApp"

}

9. HTTP_REQUEST_DATA (Payload Inspection)

Check POST data

when HTTP_REQUEST_DATA {

if {[HTTP::payload] contains "password"} {

log local0. "Sensitive data detected"

}

}

10. HTTP_RESPONSE_RELEASE (Before Sending Response)

Modify just before sending to client

when HTTP_RESPONSE_RELEASE {

HTTP::header insert "X-Test" "Release Phase"

}

- What load balancing algo will gtm use if traffic will shift to secondary member only if primary is down .

Fallback IP / Failover (Primary–Secondary method)

Simple Explanation

GTM always sends traffic to Primary (preferred) pool/member

Only when Primary is DOWN, traffic shifts to Secondary

Table Format

Feature	Explanation

Algorithm Name	Fallback IP / Failover

Primary Behavior	Always send traffic to primary member

Secondary Behavior	Used only when primary is down

Use Case	DR (Disaster Recovery), Active-Passive setup

How it is configured (Concept)

Create Wide IP

Assign Pool with members

Set:

Primary member (Order 0 / Highest priority)

Secondary member (Lower priority)

Use Global Availability method.

- What is fragmentation

Fragmentation is the process of breaking a large IP packet into smaller packets (fragments) so it can pass through a network that has a smaller MTU (Maximum Transmission Unit) limit.

🔹 Simple Explanation

👉 If packet size > MTU → it gets split into smaller pieces
👉 These pieces are called fragments
👉 Reassembled at the destination

| Feature | Explanation |
| --- | --- |
| Definition | Splitting large packet into smaller fragments |
| Why needed | Packet size exceeds MTU |
| Where happens | At router (IPv4) |
| Reassembly | At destination host |
| Protocol | Mainly IPv4 (IPv6 avoids router fragmentation) |

- Default http version in get in monitor .

The default HTTP version used in an F5 HTTP monitor (GET request) is:

## ✅ HTTP/1.0

## F5 HTTP monitor sends request like:

## GET / HTTP/1.0

## If HTTP/1.1 is needed

## GET / HTTP/1.1

## Host: example.com

## Connection: close

| Parameter | Value |
| --- | --- |
| Default Method | GET |
| HTTP Version | HTTP/1.0 |
| Connection Type | Short-lived (no keep-alive) |

- Open ssl command

| Purpose | Command |
| --- | --- |
| Check OpenSSL version | openssl version |
| Generate Private Key | openssl genrsa -out key.pem 2048 |
| Generate CSR | openssl req -new -key key.pem -out req.csr |
| Generate Self-Signed Cert | openssl req -x509 -key key.pem -out cert.pem -days 365 |
| View Certificate | openssl x509 -in cert.pem -text -noout |
| View CSR Details | openssl req -in req.csr -text -noout |
| Check Private Key | openssl rsa -in key.pem -check |
| Verify Cert & Key Match | openssl x509 -noout -modulus -in cert.pem ¦ openssl md5openssl rsa -noout -modulus -in key.pem ¦ openssl md5 |
| Convert CRT → PEM | openssl x509 -in cert.crt -out cert.pem -outform PEM |
| Convert PEM → DER | openssl x509 -in cert.pem -outform DER -out cert.der |
| Combine Key + Cert (PFX) | openssl pkcs12 -export -out cert.pfx -inkey key.pem -in cert.pem |
| Extract from PFX | openssl pkcs12 -in cert.pfx -out cert.pem -nodes |
| Test SSL Connection | openssl s_client -connect example.com:443 |
| Check Certificate Expiry | openssl x509 -enddate -noout -in cert.pem |
| Decode Base64 | openssl base64 -d -in file.txt -out file.bin |
| Encode Base64 | openssl base64 -in file.bin -out file.txt |

- What openssl  command we will use to get certificate installed on backend pool member

To get (view) the certificate installed on a backend server/pool member, use:

openssl s_client -connect <server_ip>:443

openssl s_client -connect 10.10.10.10:443

| Purpose | Command |
| --- | --- |
| Check backend server certificate | openssl s_client -connect <IP>:443 |
| Show only certificate details | openssl s_client -connect <IP>:443 ¦ openssl x509 -text -noout |
| Check specific hostname (SNI) | openssl s_client -connect <IP>:443 -servername example.com |